EXP:4.) MATLAB SIMULATION OF SUPERHETERODYNE RECEIVER FOR AM COMMUNICATION SYSTEM WITH FREQUENCY MIXING AND FILTERING

CODE:

clc;

clear;

close all;

% Parameters

fs  = 100000;         % Sampling frequency

t   = 0:1/fs:0.02;    % Time vector

fm  = 500;            % Message frequency

fc  = 10000;          % Carrier frequency

fLO = 14550;          % Local Oscillator (IF = 4550 Hz)

% Message Signal

m = cos(2*pi*fm*t);

% Carrier Signal

c = cos(2*pi*fc*t);

% AM Signal

am = (1 + m) .* cos(2*pi*fc*t);

% Local Oscillator

LO = cos(2*pi*fLO*t);

% Frequency Mixing

mixed = am .* LO;

% IF Filter

[b,a] = butter(4, 5000/(fs/2));

IF    = filter(b,a,mixed);

% Envelope Detection

env = abs(hilbert(IF));

% Display

figure;

subplot(4,1,1)

plot(t,m)

title('Message Signal')

grid on

OUTPUT: